READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Laminated Plastic Doors” Work is shown in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - NFPA No. 80 “Standard for Fire Doors and Windows

    - National Wood Windows and Doors Association, Industry Standard I.S. 1 “Wood Flush Doors”.

    - National Wood Windows and Doors Association, “Standard Recommendations and Procedures for Factory Matching Architectural Wood and Plastic-Faced Flush Doors for Hardware.”

    - Underwriters Laboratories

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

  - QUALITY ASSURANCE

    - Standards:

      - General: Comply with the requirements of the following standards unless otherwise indicated.

      - Non-Fire-Rated Wood Doors: NWWDA Industry Standard I.S. 1 “Wood Flush Doors” of the National Wood Windows and Doors Association.

      - Fire-Rated Wood Doors: Where fire-resistant classifications are shown or scheduled for plastic-faced wood door assemblies, provide doors which comply with the requirements of NFPA No. 80 “Standard for Fire Doors and Windows” and which have been tested and rated with single-point hardware by Underwriters Laboratories. Provide a UL label on each door and panel.

  - SPECIAL WARRANTIES

    - Before final inspection, furnish the manufacturer’s warranty for each door for the life of the original installation.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - SOLID-CORE, PLASTIC-LAMINATE-FACED DOORS

    - Exposed Surfaces:

      - Plastic Laminate: NEMA LD 3, general-purpose type, 1/16-inch minimum thickness. Apply to both faces of door. Color, finish and pattern as indicated on the drawings.

See Paragraph C before proceeding

      - Vertical Edges: Paint vertical edges to match plastic laminate.

    - Door Construction: Provide door construction for non-fire-rated doors as follows:

      - Solid core of the type required by the manufacturer to meet the specified one year warranty period.

      - Concealed face panels of one or more wood plies as required by the manufacturer to meet specified warranty period.

      - Type II water-resistant bond, unless Type I required to meet specified warranty period. Adhesives shall be able to handle the temperature changes during transportation, construction and building occupancy without separation.

    - Plastic-Laminate-Edged Doors: Pre-fit and pre-machine plastic-faced doors with plastic-laminate-edge finish. At the Contractor’s option, plastic-faced doors with exposed wood edges may be fitted and machined at the factory or on the job site.

  - PREPARATION FOR HARDWARE

    - Comply with NWMA “Standard Recommendations and Procedures for Factory Matching Architectural Wood and Plastic-Faced Flush Doors for Hardware.” Comply with the requirements of final hardware schedules, hardware templates, door-frame shop drawings and other essential information required to ensure proper fit of doors and hardware.

    - Doors shall be mortised to receive hardware, as specified in Section 08 70 00 - Hardware of these specifications.

  - CUT-OUTS

    - Louvers: Door shall be cut for louvers for the insert type where indicated.

    - Glazing: Doors shall be cut for glazing where indicated. Provide metal glazing stops, factory primed.

- EXECUTION

  - INSTALLATION

    - Install doors in a manner that ensures true fit to jambs and free swing without warp, twist, or distortion. Hardware shall be installed as specified in Section 08 70 00 - Hardware.

    - Fire-Rated Doors: Install in corresponding fire-rated frames in accordance with the requirements of NFPA Standard No. 80. Provide clearance complying with the limitations of authorities having jurisdiction.

  - ADJUSTMENT AND CLEANUP

    - Operation: Re-hang or replace doors which do not swing or operate freely, as directed by the Engineer.

    - Replace doors damaged during installation, as directed by the Engineer.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2